Removing and installing steel wheel bearing housing Servicing wheel bearing housing with pressed-in wheel bearing

| - Wheel bearing housing For wheel bearing with outer-ø 85 mm, or 82 mm, respectively. It is permissible to install e.g. ø 85 on left and ø82 on right side. - ◆ Removing and installing =>Page 40-30 - Wheel bearing - ◆ Always replace - ◆ Installation position: - ‒ Large internal diameter of wheel bearing faces wheel hub - ◆ Pressing out =>Fig. 2 - ◆ Pulling off bearing inner race => Fig.3 - ◆ Pressing in => Fig. 4 |  |
| --- | --- |

| - Wheel hub - ◆ Stepped version - ◆ Pressing out =>Fig. 1 - ◆ Pressing in => Fig. 5 - Hexagon bolt, 10 Nm - Cover plate - Speed sensor - ◆ Pull out to remove - ◆ Before inserting sensor, clean fitting hole inner surface and coat with lubricating paste G 000 650. - ◆ Routing => Page 40-21 - Sleeve - ◆ Grease all around with lubricating paste G 000 650 before inserting in wheel bearing housing - ◆ Press up to stop into wheel bearing housing |  |
| --- | --- |

| → Fig.1 Pressing out wheel hub |  |
| --- | --- |

| → Fig.2 Pressing wheel bearing out of wheel bearing housing |  |
| --- | --- |

| → Fig.3 Pulling bearing inner race off wheel hub Always use puller with clamp, e.g. Kukko 204- 1 (commercially available). |  |
| --- | --- |

| → Fig.4 Pressing wheel bearing into wheel bearing housing For wheel bearing with outer-ø 82 mm use the thrust pad -3124-, for outer-ø 85 mm use thrust pad -3062-. Note: Large internal diameter of wheel bearing faces wheel hub - ‒ Press wheel bearing -A- in up to stop. |  |
| --- | --- |

| → Fig.5 Pressing wheel hub into wheel bearing - ‒ When pressing in, thrust pad -VW 447i- must only make contact with the bearing inner race. |  |
| --- | --- |