Front seat covers and upholstery Removing and installing backrest cover and upholstery with sports seat Removing Important Observe safety precautions for working on airbag => - ‒ Remove front seat => Page 72-3. - ‒ Remove backrest => Page 72-10. - ‒ Remove head restraints- => Page 72-22. - ‒ Remove backrest panel => Page 74-9. - ‒ Removing head restraint guides => Page 72-22. - ‒ If applicable, detach connector under trim for seat frame. This involves removing outer seat frame trim => Page 72-6.

| - ‒ → Pull zip -1- out of backrest cover -2-. - ‒ Pinch off upholstery staples -3- and open zip. |  |
| --- | --- |

| - ‒ → Lift backrest centre section -1- and detach it from backrest catch -2-. - ‒ Pinch off staples at cover for backrest centre section and detach cover. |  |
| --- | --- |

| - ‒ → Pinch off staples -1- at cover -2- for side upholstery. - ‒ Pull up cover over side upholstery until tensioning strap becomes visible. - ‒ Detach tensioning strap and detach cover. Installing Installation is carried out in the reverse order; note the following: - ‒ Use upholstery stapler V.A.G 1634 to attach staples to cover for side upholstery. |  |
| --- | --- |

| - ‒ → Use commercially available upholstery staple gun -1- to fasten cover for backrest centre section. - ‒ Make use of upholstery staples as specified by staple gun manufacturer. |  |
| --- | --- |

| - ‒ → Secure zip -1- with upholstery needle and thread. Important - ◆ Observe installation instructions for vehicles with side airbag =>Page 69-112. - ◆ In order to guarantee the function of the side airbag the new upholstery clips must be entered at the same place as before their removal, when working at the backrest cover. - ◆ Use is only to be made of backrest covers released for side airbags by AUDI AG. - ◆ Observe seat installation instructions for vehicles with side airbag =>Page 72-5. |  |
| --- | --- |